ITEM NO. 17 (A-127)

1.	Name of the subject/project:

Sub.:	S/R of Roads in NDMC Area

SH:	Widening & Improvement of Nazaf Khan Road including its walkway and junctions

2.	Name of the Department/departments concerned

Civil Engineering Department, Road-II Division

3.	Brief history & detailed proposal of subject/project

  - The Nazaf Khan Road is proposed to be used for Park & Ride buses from Safdarjung Airport Area to JLN Stadium as per the transport plan prepared by the Special Advisor Commonwealth Games 2010.  The Park & Ride facilities are proposed to be constructed at Safdarjung Airport which would cater to the demand of spectators from different parts of Delhi at Safdarjung Airport desiring to go to JLN Stadium via Nazaf Khan Road.

  - The width of Nazaf Khan Road is 10 M and it is proposed to be widened to 12 M to cater to both way traffic of park & ride buses from Safdarjung Airport to JLN Stadium & back.

  - Besides there is need to improve the junctions of this road so as to facilitate smooth passage of park and ride buses from Safdarjung Airport to JLN stadium & back.  This road would be used during Commonwealth Games 2010 and would have a legacy value thereafter.

  - The precast RCC/CC footpath of this road was constructed more than 13 years ago.  The condition of footpath has deteriorated due to wear & tear, cuttings by various utility providers and has outlived its life.  Besides due to widening of Road the footpath would have been reconstructed.  So there is need to reconstruct the footpath & widen the road on priority in view of Commonwealth Games 2010.

  - Accordingly Preliminary Estimate has been prepared for Rs.1,20,06,000/- which has been checked by planning and concurred by Finance Department.

4.	Detailed proposal on the subject/project

- Widening of Nazaf Khan Road by one Metre on both sides with a provision of GSB & DBM layer and thereafter on entire surface a 40 mm thick Bituminous Concrete layer would be provided.  The Bituminous Concrete has already been taken up separately in RIP (Phase-II) for existing road width portion.

- Red & White sandstone on footpath.

- The scope of work also includes provision of bell mouths, bollards, cobble stone, pavers on approaches to footpaths in the form of table top crossing.

5.	Financial implications of the proposed subject/project:

The Financial implications of the scheme works out to Rs.1,20,06,000/-.

6.	Implementation schedule with timelines for each stage including internal processing.

The schedule time for completion of work is Eight months after award of work.

7.		Comments of the Finance Department on the subject with diary no. & date.

The finance vide diary No. 111/Finance/R-Civil dated 28 Jan.’09 has concurred the Preliminary Estimate for Rs. 1,20,06,000/- subject to clearance from CRRI and any other authority including traffic before accord of technical sanction to the scheme and placing on record the orders of the Competent Authority in which the road in question has been decided to be a road enroute of CWG activities.

8.	Comments of the Department on comments of Finance Department.

The recommendations of CRRI would be incorporated at Detail Estimated/Technical Sanction stage.  The traffic plan being finalized by Special Advisor CWG 2010 in which view of Traffic Police have also been taken would be placed on record after its finalization.

9.	Legal Implications of the subject/project

There are no legal implications.

10.	Details of previous Council Resolutions, existing Law of Parliament and Assembly on the subject:

NIL

11.		Comments of the Law Department on the subject :

No comments

12.	Comments of the Department on the comments of the Law Department

No comments.

13.	Certification by the Department that all Central Vigilance Commission (CVC) guidelines have been followed while processing the case

Certified that necessary guidelines of CVC would be followed while processing the tender case.

14.	Recommendations :

The case is placed before the Council for

    - Consideration & accord of Administrative Approval & Expenditure for Rs.1,20,06,000/- to the Preliminary Estimate for the work of “Widening & Improvement of Nazaf Khan Road including its walkway and junctions.

    - To initiate further action in anticipation of confirmation of Minutes by the Council.

COUNCIL’S DECISION

Resolved by the Council to accord administrative approval and expenditure sanction to the preliminary estimate amounting to Rs.1,20,06,000/- for the work of “Widening & Improvement of Nazaf Khan Road, including its walkway and junctions”.

It was also resolved that further action in the matter be taken in anticipation of confirmation of the Minutes by the Council.