ITEM NO. 03 (A-56)

1.	Name of the subject/project

Sub:	S/R of roads in NDMC area

SH:	Upgradation & Improvement of Environs of Khan Market (Phase-I)

2.	Name of the Department

Civil Engineering Department, Road-II Division.

3.	Brief history of the subject/project

- There is an outstanding demand of Khan Market Traders Association for upgradation & Improvement of environs of the market.  Several discussions were held on this matter with all concerned and it was decided during a meeting with Chairman on 25 Nov.’08 that upgradation & improvement plan of Environs of Khan Market i.e. improvement of corridor, parking, signages and middle service lane be taken up on fast track.  There is also a need to discipline the shop /building signages alongwith other development works.

- It is therefore proposed to develop Environs of Khan Market in two phases.  Improvement of verandah in front of shops, parking area adjoining carriageway and improvement of drainage etc. would be taken up in phase-I.

- Improvement & upgradation of middle lane between two blocks will be taken up under phase-II of the project as extensive discussions are required to be held with various service providers like MTNL, Airtel, IGL, & Electrical Engineering Department, Water Supply & sewerage departments because all services are laid in the middle lane only and there is requirement to design a Common Utility Duct in this middle lane.  Construction of Common Utility Duct means there would be requirement of laying of all the services new to avoid frequent cutting of middle lane for repair and maintenance of these services.

- Accordingly Preliminary Estimate for Rs.1,59,08,000/- has been prepared by the field staff and has been checked by the Planning.  Finance Department has also concurred the Preliminary Estimate.

4. 	Detailed proposal on the subject / project

- Brickwork in toe wall of the verandah, PCC 1:5:10.

- Ready Mix Concrete in the carriageway with granite cobbles on one side towards the shops and concrete pavers towards the wall after scarifying the existing Bituminous surface. The concrete pavers are proposed to be laid towards the wall as there are lots of services running below it & same pattern would be followed on other sides of Market also.

- Granite in flooring and cladding.

- Precast concrete Kerbstone & interlocking pavers, 80 mm thick.

- Installation of general road & directional signages & construction of aesthetically designed planters in front of parking (Opposite Lok Nayak Bhawan).

- Rain water pipes, electric & communication cables are proposed to be laid in pipes under the floor/ road.

- Construction on gully chambers, improvement of drainage etc.

5.  	Financial implications of the proposed project

The financial implications of the proposal works out to Rs.1,59,08,000/-.

6.  	Implementation schedule with time limits for such stage including internal processing

The schedule period of completion of work is nine months after award of work.

7.  	Comments of finance department on the subject with diary No. & date

The finance department vide diary No. 1345/Finance/R-Civil 01 Jul.’09 has concurred the proposal for Rs.1,59,08,000/- subject to following:-

- Adequacy of funds.

- Polished granite to be used on footpath is user friendly for all age groups and genders.

- Scope of work as stipulated in the estimate will not come/ encounter in the execution of 2nd phase of improvement.

8.	Comments of department on the comments of finance Department

- Funds are available against G.1.3/388.52/ Old 288.69 “Improvement to footpath in front of shops at Khan Market” with a provision of Rs.5 lacs.  Remaining funds would be sought in R.E. 2009-10.

(b) 	Polished granite to be used on footpath is user friendly for all age groups and genders with a provision of bands of sand blasted granite.

(c)	It is certified that scope of work in the Estimate will not come/ encounter in execution of second phase of improvement.

(d)	The agenda with replies to above observations of finance was sent to Finance Department & Finance Department has got no further comments to offer.

9.	Legal Implications of the subject/Project

NIL

10.	Details of previous council resolution existing law of Parliament and Assembly on subject

NIL

11.	Comments of Law Department on subject

No law point at this stage is involved.

12.	Comments of the department on the comments of Law department

No Comments.

13.	Certification by the department that all Central Vigilance Commission (CVC) guidelines have been followed while processing the case

Certified that all necessary guidelines of CVC would be followed while processing tender case.

14.	Recommendations

The case is placed before the Council for consideration and

      - Accord of Administrative Approval & Expenditure Sanction for Rs.1,59,08,000/- for the work of Upgradation & Improvement of Environs of Khan Market.

      - To initiate further action in anticipation of confirmation of Minutes of the Council meeting.

COUNCIL’S DECISION

Resolved by the Council to accord administrative approval and expenditure sanction amounting to Rs.1,59,08,000/- for the work of “S/R of roads in NDMC area.  SH.: Upgradation & Improvement of Environs of Khan Market”.

It was also resolved that further action in the matter be taken in anticipation of confirmation of the minutes by the Council.